# Thomas Doyle - 20555006

# Drawing Robot Software Planning

# Outline of the Problem to be Solved

A program is required to read text from a file, process it, and output corresponding G-Code instructions to an Arduino that controls a drawing robot. The Arduino will already be programmed to interpret these G-Code commands to move the robot’s pen.

The robot must draw each character using a font defined in the SingleStrokeFont.txt file provided. The height of the drawn text must be determined by user input, constrained between 4 mm and 10 mm, and the text must fit within a 100 mm width without breaking words across lines.

The program must meet the following requirements:

- Font Data Handling

  - Read all font data from SingleStrokeFont.txt and store it entirely in memory using an appropriate data structure.

- Text Scaling

  - Obtain the desired letter height (between 4 mm and 10 mm) from user input.

  - Apply an appropriate scaling factor to the font data so that the letters are drawn at the correct height.

- Text Processing

  - Process a text file of any length.

  - Read each word in sequence, generate the corresponding G-Code and send it to the Arduino before processing the next word.

  - Ensure that text fits within a width of 100 mm without breaking words.

- G-Code Generation

  - Translate each word into a sequence of G-Code commands that instruct the robot to:

    - Raise the pen.

    - Lower the pen.

    - Move the drawing arm to the specified X and Y coordinates according to the scaled font data.

  - The origin of each letter should correspond to the last point defined in the font data.

  - There should be 5mm of space between each line of text that the robot draws.

  - The code must interpret LF (ASCII 10, “\n”) and CR (ASCII 13, “\r”) characters to reset the X-offset and adjust the 
Y-offset for new lines.

  - The code should wait for the confirmation message “ok\n” from the Arduino before sending each subsequent command.

# Key Data Items

| Name | Data type | Rationale |
| --- | --- | --- |
| Font data | 3D integer array | It is convenient to be able to access the font data for each letter from a specific memory location. This can be achieved using a 1D array of letters. Each letter is itself a 1D array of instructions (the first element will contain the number of instructions). Each instruction is a 1D array of integers. Therefore, the whole structure will be a 3D array. The data in SingleStrokeFont.txt is in integer form so it is convenient to keep it that way. |
| Text height | Float | Storing the text height as a float gives the user more precise control over the height of the text drawn by the robot than if the data was stored as an integer. |
| Words from text file | String | Strings read from the text file are lists of ASCII values by default. This is convenient as the ASCII values can be used directly as the indices for the font data array. This means no processing is required to convert the data read from the file into another data type. |
| G-Code instructions | Array of strings | The Arduino receives G-Code instructions as strings, so it is sensible to store the instructions for a given word as a string array where each element is an instruction. |
| Pen Position | Two floats | One float for the X position and one for the Y position |

# Function Declarations

int readFontData(

const char* fileName,

int fontDataDestination[numCharsDefined][maxInstructions][3]

)

Parameters:

fileName – the name of the file storing the font data e.g. “SingleStrokeFont.txt”

fontDataDestination – the 3D array that the font data should be stored in. Its dimensions are determined by the number of characters defined in the font data text file, the maximum number of instructions for any character and the number of integers per instruction (3).

Return value – returns 0 if successful, returns 1 if the file could not be opened

This function takes the movements data in the named file and stores it in the destination array, in the format described in Key Data Items.

int readNextWord(

char* currentWord, wordBufferSize, FILE* textFile

)

Parameters:

currentWord – a pointer to the buffer that the next word is to be stored in. The size of the buffer should be much longer than the longest word reasonably expected.

textFile – a pointer to the text file.

wordBufferSize – the size of the buffer that the next word is to be stored in.

Return value – returns 1 if the end of the file has been reached. Returns 2 if the word was too long for the buffer. Returns 0 otherwise.

This function reads the next successive word as a string and stores it in wordBuffer each time the function is called. The function appends “\0” to the end of the string so the end of the word is easily identifiable. If newline characters “\r\n” (equivalent to CR LB) or punctuation are at the end of the word in textFile, these characters are treated as part of the word and appended (before “\0”).

void wordToGCode(

char* currentWord,

int charCount

char GCodeInstructions[maxInstructions][maxInstructionLength],

char fontData[numCharsDefined][maxInstructions][3]

float letterHeight,

float* penX, float* penY

)

Parameters:

currentWord – a pointer to the buffer that the current word is stored in.

charCount – The number of characters in the word

GCodeInstructions – the array that the G-Code instruction strings are to be stored in.

fontData – the array that stores the font data for each character.

letterHeight – the height of the letters to be drawn.

penX – a pointer to the X-position of the pen.

penY – a pointer to the Y-position of the pen.

This function converts the current word into G-Code instructions in string format. These strings are stored in the GCodeInstructions array.

void sendCommandList(

char GCodeInstructions[maxInstructions][maxInstructionLength]

)

Parameters:

GCodeInstructions – the array that the G-Code instruction strings are stored in.

This function sends the G-Code commands stored in GCodeInstructions one by one, waiting for the approval message, “ok”, from the Arduino between each send.

# Testing Information

| Function | Test Case | Test Data | Expected Output |
| --- | --- | --- | --- |
| readFontData | 1. Pass a file that does not exist for fileName 2. Pass a valid fileName which references a truncated version of the font data file | 1. fileName = “a.txt” where a.txt does not exist. 2. fileName = “SingleStrokeTrunc.txt” where SingleStrokeTrunc.txt is a file containing only font data for the 1st two characters | 1. Returns 1 2. Returns 0. The 3D array is populated in accordance with the format described in Key Data Items. |
| readNextWord | 1. Pass a text file which contains two words, separated by a space (“ “). Run the function twice, checking the output each time. 2. Pass a text file containing two words, separated by a newline. Run the function twice, checking the output each time. 3. Use a very small word buffer and a text file with one very long word. 4. Pass an empty text file. | 1. Text file: “Hello World!” 2. Text file: “Hello World!” 3. Text file: “abcdefghijklmnopqrstuvwxyz12345678910” 4. Text file: “” (empty) | 1. First time: The word buffer contains “Hello” Returns 0 Second time: The word buffer contains “World!” Returns 1 2. First time: The word buffer contains “Hello\r\n” Returns 0 Second time: The word buffer contains “World!” Returns 1 3. The word buffer is not changed. Returns 2 4. The word buffer is not changed. Returns 1 |
| wordToGCode | 1. Pass a word with a known G-Code command set with letterHeight set to 10mm. 2. Pass the same word but with letterHeight set to 4mm | 1. currentWord = “Hi” letterHeight = 10.0f 2. currentWord = “Hi” letterHeight = 4.0f |  |
| sendCommandList | 1. Pass three commands which draw a right-angled triangle with perpendicular side lengths of 10mm. |  | 1. The robot draws a right-angled triangle with side lengths of 10mm |
| main | 1. Pass a text file with two words separated by a space (“ “) at 10mm letter height. 2. Pass a text file with a long sentence but no newlines at 10mm letter height. 3. Pass a text file with 2 words separated by a newline at 10mm letter height. 4. Pass a text file with multiple long sentences and newlines at 10mm letter height. 5. Pass a text file with multiple long sentences and newlines at 4mm letter height. | 1. Text file: “Hello World!” Letter height input: 10 2. Text file: “The quick brown fox jumped over the lazy dog.” Letter height input: 10 3. Text file: “Hello World!” Letter height input: 10 4. Text file: “Far out in the uncharted backwaters of the unfashionable end of the western spiral arm of the Galaxy lies a small unregarded yellow sun. Orbiting this at a distance of roughly ninety-two million miles is an utterly insignificant little blue-green planet whose ape-descended life forms are so amazingly primitive that they still think digital watches are a pretty neat idea.” Letter height input 10 5. Text file: same as (4.) Letter height input: 4 | For each of these tests the robot is expected to write the text at the specified text height. Any newlines in the text file should result in a newline in the drawing. If the next word would be drawn outside the 100mm width allowed, the robot should start drawing on the next line. When finished the robot should return the pen to (0,0). |

Extend table as required. Note that ‘Function’ includes main()

# Flowchart(s)

Included as a separate PDF in the GitHub repository.

# AI Statement

AI was not used in the production of this project plan.